Packet Tracer - Implement Physical Security with IoT Devices

Additional Note: Yellow highlighted text indicates all of my answers.

(672021159)

Objectives

Part 1: Connect IoT Devices to the Network

Part 2: Add IoT Devices to the Registration Server

Part 3: Explore IoT Security Device Functionality

Background / Scenario

In an attempt to increase the physical security of your home, you have decided to install some Internet of Things (IoT) devices to allow you to remotely monitor your home while you are away. In this Packet Tracer activity, you will install an IoT device to enhance home security. You will then configure all the IoT devices to connect to the wireless network and communicate with a remote IoT registration server.

Instructions

Part 1: Connect IoT Devices to the Network

In this part, you will connect IoT devices to the wireless network and implement MAC address filtering.

Step 1: Connect the Siren to the Door

a.     Click Home and locate the Home_Siren and Home Doors in the living room.

b.     In the toolbar at the bottom, click Connections > IoT Custom Cable, which is the second to last option.

c.     Click Home_Siren > DO interface, and then click Home Doors > DO interface.

d.     Press and hold the ALT key and then click the door to open and close it. Notice that opening the door will now activate the siren.

Step 2: Associate all IoT Devices to HomeNet wireless network.

The Home Wireless Router is pre-configured to use WPA2-PSK and MAC Address filtering to control who can associate with the network and which devices can use the network to transfer traffic. All new devices must conform to the current configuration.

a.     Locate the four wireless IoT devices:

o      Home_Siren

o      Home Doors

o      Home_Motion_Sensor (home office windows)

o      Home_Webcam (home office bookshelf)

b.     Click Home_Siren, and then Config tab > Wireless0.

c.     Configure the SSID and Authentication mode.

o      SSID: HomeNet

o      Authentication: WPA2-PSK

o      Pass Phrase: ciscorocks

d.     In IP Configuration, click DHCP. Verify the device received an IP address from the 192.168.0.0/24 network.

Note: It may be necessary to toggle between Static and DHCP to force Packet Tracer to converge on your settings.

e.     Record the MAC address for the IoT device. Format the addresses with colons between each two hexadecimal numbers instead of a period between each four hexadecimal numbers. This format is required for the next step when you will apply MAC address filtering.

Questions:

o      Home_Siren

00:90:2B:C8:C6:E2

o      Home Doors

00:0A:F3:DB:2E:37

o      Home_Motion_Sensor

00:40:0B:39:21:24

o      Home_Webcam

00:D0:58:D1:C4:11

f.      Repeat these steps for each IoT device.

Step 3: Configure MAC address filtering to allow the IoT devices.

a.     Click Home Office PC > Desktop tab > Web Browser.

b.     Login to the Home Wireless Router at 192.168.0.1. Use admin as both the username and password.

c.     Click Home Wireless Router > GUI tab.

d.     Click Wireless, and then Wireless MAC Filter.

e.     Verify that the filter is enabled for the 2.4 GHz wireless port and that it is set to permit PCs to access the wireless network.

f.      Add the four IoT device MAC addresses to the table.

g.     Scroll to the bottom and click Save Settings.

h.     The Home Wireless Router will reboot. Close the Web Browser, and then click IP Configuration. If necessary, toggle the DHCP and Static buttons to renew the IP configuration so that Home Office PC has an IP address from the 192.168.0.0/24 network.

Part 2: Add IoT Devices to the Registration Server

In this part, you will sign up for an account with a IoT remote monitoring service. You will then register the IoT devices to communicate with the service. This will allow you to remotely monitor the IoT devices through a web portal.

Step 1: Create an account on the ISP IoT Registration Server.

a.     On Home Office PC, close IP Configuration, and then click Web Browser.

b.     Navigate to http://10.3.0.125.

c.     Click Sign up now and create a new account with username HomeUser and password Pa$$w0rd.

Step 2: Register each IoT device with the Registration Server

Each of the four IoT devices must be registered with a registration server to remotely monitor and manage the device. Repeat the following steps for each IoT device.

a.     Click the device > Config tab > Settings under GLOBAL.

b.     Scroll to the IoT section at the bottom of the page and enter the following information:

o      Select Remote Server

o      Server Address: 10.3.0.125

o      User Name: HomeUser

o      Password: Pa$$w0rd

c.     Click Connect button. The button will change to Connecting and then to Refresh in a few seconds. The IoT device is now registered with the server.

Note: If it does not change to Refresh then you may have entered the information incorrectly. Therefore, re-enter the information and repeat this step.

d.     Repeat the above steps to register all the IoT devices.

Step 3: Verify all devices are registered.

a.     Return to the Web Browser on the Home Office PC. If necessary, browse to 10.3.0.125 and login with username HomeUser and password Pa$$w0rd.

b.     Verify all four IoT devices are registered on the server.

Open configuration window

Part 3: Explore IoT Security Device Functionality

In this part, you will explore the functionality of the IoT devices and monitor their states on the IoT Registration Server.

Step 1: Observe and control IoT devices from the registration server.

a.     From the Web Browser on the Home Office PC, click each section for each IoT device to display device details.

Question:

What is the status of each device?

Semua perangkat IoT menampilkan status “On”, menandakan bahwa semuanya sudah aktif dan terhubung ke server registrasi IoT dengan baik.

b.     Click the red rectangle under Home Doors to activate it.

Question:

What happened?

Saat tombol Home Doors diaktifkan (berwarna merah diklik), indikator untuk pintu dan sirene berubah menjadi hijau secara bersamaan. Ini menandakan bahwa kedua perangkat tersebut aktif dan saling terhubung. Pintu terbuka, dan sirene menyala.

c.     Click the door rectangle again to deactivate the device.

Question:

What happened?

Ketika tombol pintu diklik lagi untuk menonaktifkannya, warna indikator pada sirene dan pintu sama-sama berubah menjadi merah. Ini menunjukkan bahwa kedua perangkat sudah dalam keadaan nonaktif (pintu tertutup, sirene berhenti berbunyi).

d.     Click the green rectangle for Home_Webcam.

Question:

What happened?

Ketika ikon webcam diklik (warna hijau), tampilan video langsung berhenti. Artinya kamera berhasil dimatikan melalui kontrol jarak jauh.

e.     Click the green rectangle again to turn on the Home_Webcam.

Ketika webcam diaktifkan kembali, video feed langsung muncul di server registrasi. Ini memastikan fungsi kendali jarak jauh webcam berjalan normal.

f.      Click the red circle for Home_Motion_Sensor. Nothing will happen because the motion sensor only sends information to the registration server. This is an example of a device that can be monitored only.

Tidak ada aksi langsung di tampilan server karena sensor gerak hanya berfungsi mengirimkan data, bukan menerima perintah. Artinya perangkat ini hanya bisa dimonitor, bukan dikontrol secara langsung.

Step 2: Interact with the sensors in the home.

In Packet Tracer, the IoT devices can normally be activated by holding down the ALT key and then clicking the device (ALT-click). The Motion sensor is activated by holding down the ALT key and moving the mouse over the sensor. Have the IoT registration server screen visible for these actions.

a.     ALT-Click on the door.

Question:

What happened?

Ketika pintu diaktifkan dengan kombinasi ALT + klik, indikator untuk pintu dan sirene di layar berubah warna, menandakan keduanya aktif. Notifikasi ini juga muncul di dashboard server, memperlihatkan bahwa koneksi real-time antarperangkat berfungsi.

b.     ALT-Click on the webcam.

Question:

What happened?

Saat webcam di-ALT-klik, lampu indikator pada kamera padam, artinya kamera dalam posisi mati. Status tersebut juga tercermin pada tampilan server yang memperlihatkan webcam sudah dinonaktifkan.

c.     Depress the ALT key and move your mouse over the motion sensor.

Question:

What happened?

Ketika kursor digerakkan di atas sensor gerak sambil menekan ALT, lampu aktivitas pada sensor menyala yang menandakan adanya gerakan yang terdeteksi. Server juga memperlihatkan status aktif secara bersamaan.

Reflection Questions

- What advantages do IoT enabled devices provide for home security?

Perangkat IoT memberikan kemudahan instalasi dan konfigurasi karena bisa langsung terhubung ke jaringan rumah yang sudah ada. Selain itu, pengguna bisa memantau kondisi rumah secara real-time dari jarak jauh, meningkatkan kenyamanan sekaligus keamanan.

- What is the advantage of using a registration server for IoT enabled devices?

Server registrasi berfungsi sebagai pusat kendali dan monitoring jarak jauh. Dengan server ini, pengguna dapat memantau, mengontrol, atau menonaktifkan perangkat IoT (seperti kamera dan alarm) kapan pun dan dari mana pun melalui koneksi internet.

- Search the internet to find what types of IoT enabled devices are used for home security?

Beberapa perangkat IoT yang umum digunakan untuk keamanan rumah antara lain:

- Kamera pengawas (CCTV / webcam)

- Sensor gerak

- Alarm pintu dan jendela

- Detektor asap dan api

- Sensor kebocoran air

- Smart lock (kunci digital)

Perangkat-perangkat ini bekerja sama untuk memberikan sistem keamanan rumah yang lebih cerdas dan responsif.

End of document